# Edge Safe Guard – Technical Documentation

### Model Development Track, HackerEarth × Edge Impulse Hackathon 2025

## 1. Overview

Edge Safe Guard is an edge-deployed safety-helmet detection system designed for construction and manufacturing sites.
The project builds a lightweight computer-vision model in Edge Impulse Studio, targeting real-time performance on resource-constrained hardware.
This document explains the model pipeline, dataset preparation, training iterations, and deployment workflow.

## 2. Problem Definition

Lack of proper safety-helmet usage contributes significantly to workplace injuries.
Conventional CCTV systems do not detect violations autonomously, and cloud-based solutions introduce latency, cost, and privacy concerns.
This project demonstrates an on-device inference system that performs detection without network dependency.

## 3. Dataset Preparation

### 3.1 Dataset Sources

The dataset was built using a combination of:

- Open-source “Safety Helmet / Hardhat” datasets with permissive licensing.

- Supplemental manual data collection (images taken at home/work environments).

### 3.2 License Compliance

All datasets were verified to be:

- Open-source

- Allowed for commercial + redistribution use

- Suitable for model training

### 3.3 Dataset Structure

dataset/

├── raw/

├── processed/

└── metadata/

Raw images were normalized and resized during Edge Impulse ingestion.
Bounding-box annotations were imported in the COCO JSON format.

## 4. Data Processing

Inside Edge Impulse Studio:

### 4.1 Image preprocessing

- Resize: 320×320

- Color depth: RGB

- Normalization: 0–255 → 0–1

- Augmentation:

  - Random flip

  - Random brightness

  - Random rotation

  - Crop jitter

The objective was to counter dataset imbalance and improve generalization.

## 5. Model Architecture

### 5.1 Impulse Structure

- Input block: 320×320 RGB

- Processing block: Image DSP (Edge Impulse built-in)

- Learning block: FOMO (Faster Objects, More Objects) MobileNetV2-0.35

### 5.2 Why FOMO?

- Optimized for edge devices

- Faster than YOLO-class detectors

- Significantly smaller memory footprint

- Suitable for small objects (helmet detection)

## 6. Training Setup

### 6.1 Hyperparameters

| Parameter | Value |
| --- | --- |
| Epochs | 75 |
| Batch size | 32 |
| Learning rate | 0.001 |
| Optimizer | Adam |
| Split ratio | 80% train / 20% validation |
| Quantization | INT8 |

### 6.2 Hardware Optimization

Edge Impulse provides an on-device benchmark.
The model was optimized with:

- EON Compiler

- Quantized INT8 parameters

These produced a final .wasm package suitable for browser-based inference.

## 7. Model Performance

### 7.1 Metrics

The final model achieved:

- mAP: ~40%

- Precision/Recall: Balanced but limited by dataset variability

- Confusion matrix: Extracted from EIS evaluation tools

### 7.2 Performance Discussion

Due to dataset diversity (multiple helmet shapes/colors/backgrounds), accuracy is moderate, but:

- The model performs reliably on clear frontal frames

- False negatives reduce when lighting is consistent

- Augmentation helps model generalization

## 8. Deployment

### 8.1 Selected Deployment Target

WebAssembly (Browser, SIMD)
Reason:

- Works on any OS (Windows / Mac / Linux)

- No installation required

- Ideal for hackathon demonstration

### 8.2 Deployment Files

Generated by Edge Impulse Studio:

deployment/

└── webassembly/

├── edge-impulse-standalone.js

├── edge-impulse-standalone.wasm

├── wrapper.js

├── model_metadata.json

### 8.3 Frontend Application

A minimal camera-inference setup using:

- HTML5 Video

- JavaScript WebAssembly loader

- Canvas for per-frame rendering

This allowed real-time detection in the browser without local installation.

## 9. Application Workflow

- Camera feed is captured

- Frame is drawn onto an HTML canvas

- Pixel data is extracted

- Passed to runner.classify()

- Model returns detection results

- (Optional) Bounding boxes are drawn (if supported by deployment mode)

## 10. Limitations

- FOMO is best for small-object detection, but performance drops on:

  - Low-light scenes

  - High motion blur

- Lack of bounding-box overlay support in WebAssembly runtime limits visualization

- Dataset size was smaller than ideal for high-accuracy detection

## 11. Future Improvements

- Expand dataset with industrial site images

- Use synthetic image generation to improve helmet variety

- Add bounding-box rendering pipeline using custom post-processing

- Deploy to real edge hardware (Nvidia Jetson / Raspberry Pi)

- Improve dataset balance for “no helmet” class

## 12. Conclusion

The Edge Safe Guard system demonstrates how a lightweight, resource-efficient vision model can be built entirely within Edge Impulse Studio and deployed in a browser-based WebAssembly environment.
Despite moderate accuracy, the system highlights a complete edge-AI pipeline, from dataset ingestion to real-time inference.

## 13. Repository Contents

/

├── dataset/

├── deployment/

├── docs/

│   ├── README.md

│   ├── technical_report.md

│   └── architecture_diagram.png

├── webapp/

│   ├── index.html

│   ├── camera.js

│   ├── detector.js

└── model/

## 14. Additional References

- Edge Impulse Documentation

- COCO dataset structure guidelines

- WebAssembly SIMD specifications